Introduction

AEC 원리

CT 영상의 잡음(noise) 수준은 촬영 부위를 투과하는 X선의 감쇠(attenuation) 정도에 따라 결정되며, 동일한 관전류(tube current, mA)로 촬영하더라도 환자의 체격이나 부위별 두께가 다르면 잡음 수준이 균일하지 않게 된다. 이를 보정하기 위해 현대 다중검출기 CT는 자동노출조절(automatic exposure control, AEC)을 이용하여, 사전에 획득한 localizer(스카우트) 영상에서 얻은 부위별 X선 감쇠 정보를 바탕으로 사용자가 지정한 목표 영상잡음을 유지하도록 관전류를 실시간으로 조정한다. AEC는 촬영 각도에 따라 관전류를 바꾸는 각(angular, x-y) 변조, 환자의 머리에서 발 방향(craniocaudal, z축)을 따라 관전류를 바꾸는 종축(longitudinal, z) 변조, 그리고 두 방식을 결합한 x-y-z 복합 변조로 구분된다.

이때 부위별 감쇠 정보는 본 촬영 직전에 획득하는 저선량 예비영상, 즉 localizer(scout) 영상으로부터 추정된다. Localizer는 관전류를 낮게 고정한 상태로 환자를 전후방(AP) 또는 측방(lateral) 투영으로 스캔하여 얻는 단순 투과영상으로, 이 영상의 화소 강도 분포로부터 z축 위치별 대략적인 X선 감쇠량을 산출할 수 있다. 시스템은 이 감쇠 추정치와 사용자가 지정한 목표 영상잡음 지표(noise index, 표준편차, 기준 mAs 등)를 결합하여 z축 위치별 관전류 프로파일을 사전 계산하며, 본 촬영이 시작되면 이 프로파일을 따라 관전류를 실시간으로 조정한다(일부 벤더의 경우 촬영 초반 half rotation 동안의 실측 감쇠값으로 추가 보정을 수행한다).

복부 CT에서 관전류 변조는 z축(longitudinal) 위치에 따른 두께 변화뿐 아니라 단면 내 좌우·전후 비대칭(각/x-y 변조)까지 함께 반영하는 x-y-z 복합 변조로 구현되는 경우가 많다. 감쇠가 상대적으로 낮은 부위(예: 연부조직 위주의 중간 복부)에서는 관전류를 낮추고, 감쇠가 높은 부위(예: 치밀골이 많은 골반)에서는 관전류를 높여 촬영 전 구간에서 일정한 영상 품질을 유지한다. 그 결과 촬영 진행 방향을 따라 기록되는 관전류 값의 궤적, 즉 tube-current modulation curve는 해당 z축 위치에서 환자 단면의 감쇠 특성 - 곧 체형 및 체구성(body habitus) - 을 반영하게 된다. 이러한 원리에 근거해 관전류 값과 체중 간의 강한 상관관계를 보고한 선행 연구도 있으며, 이는 AEC 곡선이 체형의 대리 지표(surrogate marker)로 활용될 수 있음을 뒷받침한다.

본 연구에 포함된 CT 장비는 x-y-z 복합 변조(combined modulation)를 구현하고 있어, DICOM 헤더에서 추출한 관전류 값(XRayTubeCurrent)은 순수한 z축(longitudinal) 변조만의 결과가 아니라, 단면 내 각도별 비대칭을 반영하는 각(x-y) 변조와 두부-족부 방향의 종축(z) 변조가 결합된 combined modulation의 출력값이다. 다만 이 값은 슬라이스(회전) 당 하나로만 기록되므로, 동일 회전 내 각도별 세부 변동은 별도로 구분되지 않고 이미 해당 회전에 대한 대표값으로 통합되어 있다. 따라서 본 연구의 AEC-128 곡선은 두부-족부 방향 위치에 따라 값이 변화하는 형태를 취하지만, 각 지점의 값 자체는 z축뿐 아니라 단면 비대칭 정보까지 반영한 x-y-z 복합 변조의 결과값이다.

본 연구는 이러한 관전류 변조 원리를 이용해, DICOM 헤더에 기록된 실측 관전류(XRayTubeCurrent) 값을 간(liver) 하단부터 골반뼈(hip) 상단까지의 구간에서 추출하고, 환자 간 상이한 슬라이스 수(101-238매)를 128개 지점으로 선형보간하여 표준화한 곡선인 AEC-128을 구성하였다. 이는 CT 영상 자체의 화소값을 별도로 분석하지 않고도, 촬영 과정에서 자동 기록되는 관전류 정보만으로 체형 관련 정보를 opportunistic하게 추출하는 접근에 해당한다.

관전류 대신 관전압(kVp)이나 노출시간(회전시간)을 조절하여 동일한 목적을 달성할 수도 있으나, 각각의 한계로 인해 관전류 변조가 AEC의 표준 구현 방식으로 채택되어 있다. 관전압을 변경하면 X선의 에너지 스펙트럼, 즉 빔의 질(beam quality) 자체가 달라져 조직 간 대조도, 조영증강 정도, 빔경화(beam hardening) 아티팩트 양상까지 함께 변화하므로 검사마다 별도의 재보정이 필요하다. 노출시간(갠트리 회전시간)을 늘리면 촬영 시간이 길어져 호흡 및 장관 연동운동에 의한 움직임 흐림(motion blur)의 위험이 커지며, 회전 속도에는 기계적으로 정해진 제약이 있어 임의로 조절하기도 어렵다. 반면 관전류는 빔의 에너지 스펙트럼이나 스캔 속도를 변화시키지 않고 X선 광자 수(선량)만을 근사적으로 선형에 가깝게 조절할 수 있어, 영상의 대조도·시간분해능 특성을 그대로 유지한 채 환자 단면의 두께 차이만을 보정하는 변수로 활용된다. 본 연구의 포함 기준상 모든 대상자가 100 kVp로 고정 촬영되었으므로(Materials 참조), 환자 간 관전류 값의 차이는 촬영 프로토콜 차이가 아닌 체형 차이만을 반영한다고 볼 수 있다.

Materials

This retrospective cohort study comprised two independent institutional cohorts: an internal cohort from Gangnam Severance Hospital and an external cohort from Sinchon Severance Hospital. Because the two cohorts were drawn from different hospitals and different scanner populations, the model trained on the internal cohort was applied, without retraining, to the external cohort to assess external validity. CT examinations for both cohorts were performed between January 2018 and June 2020.

Inclusion criteria were as follows:

1. Both clinical data and an abdominal CT examination were available for the same patient.

2. Only CT examinations acquired at a tube voltage of 100 kVp were included; in the internal cohort, patients were further restricted to those scanned on the four most commonly used CT scanner models at that site (Siemens Sensation 64, GE Revolution CT, Philips Ingenuity Core 128, and Siemens SOMATOM Definition AS+), whereas in the external cohort no scanner-model restriction was applied except exclusion of the negligibly represented Canon scanners (n = 3).

3. Patients younger than 20 years of age were excluded from both cohorts.

After application of these criteria, the final study population consisted of 1,079 patients in the internal cohort and 922 patients in the external cohort.

Age, sex, height, and weight were abstracted from the same clinical database used for patient matching and served as the baseline clinical predictors, collectively termed clinic4. Outcomes were defined as the presence or absence of a documented diagnosis of hypertension (HTN), diabetes mellitus (DM), and chronic kidney disease (CKD) in the clinical database, rather than by dichotomizing a continuous laboratory measurement. Hypertension was present in 324 of 1,079 internal patients (30.0%) and 424 of 922 external patients (46.0%); diabetes mellitus in 202 internal patients (18.7%) and 285 external patients (30.9%); and chronic kidney disease in 83 internal patients (7.7%) and 168 external patients (18.2%).

Baseline characteristics of the internal and external cohorts are summarized in Table 1. There were no missing values for age, sex, height, weight, BMI, comorbidity, or scanner vendor in either cohort.

Table 1. Baseline Characteristics of the Patients

| Characteristic | Internal cohort (n = 1,079) | External cohort (n = 922) | p-value |
| --- | --- | --- | --- |
| Age, years, mean ± SD | 57.1 ± 12.1 | 59.5 ± 12.7 | <0.001 |
| Age, years, range | 20-91 | 20-90 | <0.001 |
| Sex, female, n (%) | 692 (64.1) | 497 (53.9) | <0.001 |
| Sex, male, n (%) | 387 (35.9) | 425 (46.1) | <0.001 |
| Height, cm, mean ± SD | 162.2 ± 8.2 | 162.5 ± 8.6 | 0.401 |
| Weight, kg, mean ± SD | 61.9 ± 10.6 | 61.8 ± 11.4 | 0.873 |
| BMI, kg/m², mean ± SD | 23.5 ± 3.1 | 23.3 ± 3.4 | 0.383 |
| Hypertension, n (%) | 324 (30.0) | 424 (46.0) | <0.001 |
| Diabetes mellitus, n (%) | 202 (18.7) | 285 (30.9) | <0.001 |
| Chronic kidney disease, n (%) | 83 (7.7) | 168 (18.2) | <0.001 |
| CT scanner vendor, n (%) |  |  | <0.001 |
| — Siemens | 560 (51.9) | 568 (61.6) |  |
| — GE | 317 (29.4) | 219 (23.8) |  |
| — Philips | 202 (18.7) | 135 (14.6) |  |

SD, standard deviation; BMI, body mass index. Continuous variables were compared with Welch's *t*-test; categorical variables were compared with the chi-square test.

During each abdominal CT acquisition, automatic exposure control (AEC) modulates the tube current (mA) along the craniocaudal (z) axis to maintain a constant image noise level; the resulting tube-current modulation curve reflects the patient's cross-sectional attenuation, and hence body habitus, at each slice position. In this study, a fixed-length, 128-point representation of this curve (AEC-128) was used as the imaging-derived predictor.

Method

AEC-128 Curve Construction

To construct the fixed-length AEC-128 feature, the axial source series was first identified for each patient, excluding coronal/sagittal reformations, scout images, and dose reports, based on slice count, DICOM ImageType, SeriesDescription, and image orientation. An open-source, deep-learning-based segmentation tool (TotalSegmentator) was then applied to localize the liver and the bilateral hip bones, with the cranial-most liver slice and caudal-most hip slice defining the upper and lower boundaries of the analysis range; segmentation was used solely for this anatomical landmark localization, while tube-current values themselves were obtained directly from the DICOM header (XRayTubeCurrent). Tube-current values within this pubis-to-liver range were extracted in craniocaudal order, and because the resulting slice count varied across patients from 101 to 238 slices, each sequence was resampled by linear interpolation to a fixed length of 128 points, yielding the AEC-128 curve used as the analytic unit in subsequent modeling.

Functional Principle Component Analysis

AEC-128 곡선은 환자 1인당 128차원의 변수이나, 인접 지점 값이 서로 강한 상관관계(autocorrelation)를 갖는 함수형 자료(functional data)이다. 128개 지점을 독립 변수로 그대로 투입할 경우 다중공선성과 과적합 위험이 크므로, 함수주성분분석(functional principal component analysis, FPCA)을 적용하여 곡선의 형태 정보를 저차원으로 요약하였다. FPCA는 주성분분석(PCA)의 원리를 함수형 자료로 확장한 차원축소 기법으로, 개별 곡선을 평균곡선과 곡선 간 변동을 가장 많이 설명하는 일련의 직교 성분들의 선형결합으로 근사하며, 각 환자의 곡선은 이들 성분에 대한 투영값인 성분점수(FPC score)로 요약된다.

구체적으로 ① 내부 코호트 전체 곡선의 평균곡선 mu(z) (z = 1, ..., 128)를 산출하고, ② 각 환자 원곡선에서 평균을 차감한 편차곡선 d_i(z) = x_i(z) - mu(z)를 구하고, ③ 편차곡선들의 128×128 공분산행렬을 고유분해(수치적 안정성을 위해 실제로는 특이값분해(SVD)를 이용, scikit-learn PCA)하여 분산을 가장 많이 설명하는 순서로 정렬된 성분(고유함수, phi_k)과 고유값(lambda_k)을 얻고, ④ 각 환자의 편차곡선을 각 성분에 투영하여 성분점수 score_i,k = Σ_z d_i(z)·phi_k(z)를 산출하였다.

Figure 1. Computation of functional principal component analysis (FPCA) on the internal-cohort AEC-128 curves. (A) A random sample of patient curves (gray) and the resulting mean curve μ(z) (black). (B) Deviation curve dᵢ(z) = xᵢ(z) − μ(z) for a representative patient. (C, F) The 128×128 covariance matrix, eigen-decomposed (via SVD) to obtain (E) the eigenvalue scree plot used for elbow-based component selection (k = 3) and (G) the resulting eigenfunctions φ₁–φ₃, shown as μ(z) ± √λₖ·φₖ(z). (D) Projection of the representative patient's deviation curve onto φ₁ yields the component score scoreᵢ,₁ = Σᵩ dᵢ(z)φ₁(z) = 1,318.

내부 코호트에 PCA를 적합한 결과 PC1이 전체 변동의 84.3%를, PC1-2 누적 94.8%를, PC1-3 누적 97.4%를 설명하였다. 성분 수는 누적설명분산의 scree plot에서 elbow(Kneedle 알고리즘: 성분 수와 개별 설명분산비율을 각각 0-1로 정규화한 뒤 첫 성분과 마지막 성분을 잇는 직선까지 수직 거리가 최대인 지점을 elbow로 정의)로 결정하였으며, 그 결과가 PC3와 일치하여 3개 성분점수(PC1-PC3)를 AEC 곡선의 대표 지표로 고정 채택하였다. 개별 환자 수준의 재구성 정확도를 예시로 확인하기 위해 PC1-PC3 성분점수가 모두 크고 재구성 적합도가 상위 25% 이내인 한 환자(PC1 +1,318, PC2 +1,175, PC3 -247)를 재구성한 결과, 원곡선과의 결정계수(R²)는 0.992로 나타나 128개 지점의 원자료가 3개의 성분점수만으로 근사됨을 확인하였다.

성분(고유함수)은 내부 코호트에서만 추정하고, 외부 코호트의 곡선은 이 고정된 성분에 재학습 없이 투영하여 성분점수만 산출함으로써 두 코호트를 동일한 기준으로 비교하였다. 이는 평균·표준편차·왜도와 같은 개별 요약통계량이 포착하지 못하는 곡선의 형태학적 변이를 정량화하여 예측모델의 입력 변수로 활용하기 위함이다.